# Geogard 221

Version	1.0 / EN

Revision Date	19.09.2018

Print Date	12.12.2018

## SECTION 1: Identification of the substance/mixture and of the company/undertaking

  - Product identifier

| Product name | : | Geogard 221 |
| --- | --- | --- |

  - Relevant identified uses of the substance or mixture and uses advised against

| Use of the Substance/Mixture | : | Conservation agent (preservative) for cosmetics |
| --- | --- | --- |

  - Details of the supplier of the safety data sheet

| Company | : | Sophix Natural and Agro Products Ltd 5, Olayinka Owodunni Street, Off Sule Abore, Behind NNPC Filling Stattion, Lagos Business Telephone: +234 7061111838 |
| --- | --- | --- |
| E-mail address Responsible/issuing person | : | sophixnatural@gmail.com |

  - Emergency telephone number

| Emergency telephone number | : | +234 7061111838 |
| --- | --- | --- |

## SECTION 2: Hazards identification

| Acute toxicity, Category 4 | H302: Harmful if swallowed. |
| --- | --- |
| Acute toxicity, Category 4 | H332: Harmful if inhaled. |

Classification of the substance or mixture Classification (REGULATION (EC) No 1272/2008)

  - Label elements

Labelling (REGULATION (EC) No 1272/2008)

| Hazard pictograms | : |  |
| --- | --- | --- |
| Signal word | : | Warning |
| Hazard statements | : | H302 Harmful if swallowed. H332 Harmful if inhaled. |

| Precautionary statements | : | Prevention: P261 Avoid breathing mist/vapours/spray. Response: P301 + P312 IF SWALLOWED: Call a POISON CENTER/doctor if you feel unwell. |
| --- | --- | --- |

  - Other hazards

This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.

No hazards to be specially mentioned.

## SECTION 3: Composition/information on ingredients

3.2 Mixtures

Hazardous components

| Chemical name | CAS-No. EC-No. Index-No. Registration number | Classification | Concentration (% w/w) |
| --- | --- | --- | --- |
| Benzyl alcohol | 100-51-6 202-859-9 603-057-00-5 01-2119492630-38- XXXX | Acute Tox. 4; H302 Acute Tox. 4; H332 | >= 70 - < 90 |
| 3-Acetyl-6-methyl-2H-pyran- 2,4(3H)-dione | 520-45-6; 771-03-9 208-293-9 607-163-00-2 01-2120747930-51- XXXX | Acute Tox. 4; H302 | >= 5 - < 10 |
| Substances with a workplace exposure limit |  |  |  |

For explanation of abbreviations see section 16.

## SECTION 4: First aid measures

  - Description of first aid measures

| If inhaled | : | Move to fresh air. Consult a physician after significant exposure. If unconscious, place in recovery position and seek medical advice. If breathing is irregular or stopped, administer artificial respiration. Keep respiratory tract clear. |
| --- | --- | --- |
| In case of skin contact | : | After contact with skin, wash immediately with plenty of soap and water. If on clothes, remove clothes. In the case of skin irritation or allergic reactions see a physician. |
| In case of eye contact | : | Immediately flush eye(s) with plenty of water. Remove contact lenses. |

|  |  | Protect unharmed eye. Keep eye wide open while rinsing. If eye irritation persists, consult a specialist. |
| --- | --- | --- |
| If swallowed | : | Clean mouth with water and drink afterwards plenty of water. Never give anything by mouth to an unconscious person. If symptoms persist, call a physician. |

  - Most important symptoms and effects, both acute and delayed

| Symptoms | : | No information available. |
| --- | --- | --- |

  - Indication of any immediate medical attention and special treatment needed

| Treatment | : | Treat symptomatically. |
| --- | --- | --- |

## SECTION 5: Firefighting measures

  - Extinguishing media

| Suitable extinguishing media | : | Water spray Alcohol-resistant foam Dry chemical |
| --- | --- | --- |
| Unsuitable extinguishing media | : | High volume water jet |

  - Special hazards arising from the substance or mixture

| Specific hazards during firefighting | : | Heating or fire can release toxic gas. |
| --- | --- | --- |

  - Advice for firefighters

| Special protective equipment for firefighters | : | In the event of fire, wear self-contained breathing apparatus. Use personal protective equipment. |
| --- | --- | --- |

| Further information | : | Use water spray to cool unopened containers. |
| --- | --- | --- |

## SECTION 6: Accidental release measures

  - Personal precautions, protective equipment and emergency procedures

| Personal precautions | : | Use personal protective equipment. Ensure adequate ventilation. Use respirator when performing operations involving potential exposure to vapour of the product. |
| --- | --- | --- |

  - Environmental precautions

| Environmental precautions | : | Prevent product from entering drains. Prevent further leakage or spillage if safe to do so. |
| --- | --- | --- |

  - Methods and material for containment and cleaning up

| Methods for cleaning up | : | Contain spillage, and then collect with non-combustible absorbent material, (e.g. sand, earth, diatomaceous earth, vermiculite) and place in container for disposal according to local / national regulations (see section 13). |
| --- | --- | --- |

  - Reference to other sections

For personal protection see section 8.

For disposal considerations see section 13.

## SECTION 7: Handling and storage

  - Precautions for safe handling

| Advice on safe handling | : | Avoid formation of aerosol. Do not breathe vapours/dust. Smoking, eating and drinking should be prohibited in the application area. Provide sufficient air exchange and/or exhaust in work rooms. Dispose of rinse water in accordance with local and national regulations. |
| --- | --- | --- |
| Advice on protection against fire and explosion | : | Take precautionary measures against static discharges. |
| Hygiene measures | : | Avoid contact with skin, eyes and clothing. When using do not eat or drink. When using do not smoke. |

  - Conditions for safe storage, including any incompatibilities

| Requirements for storage areas and containers | : | Keep container tightly closed. Keep in a well-ventilated place. Electrical installations / working materials must comply with the technological safety standards. To maintain product quality, do not store in heat or direct sunlight. |
| --- | --- | --- |
| Other data | : | No decomposition if stored and applied as directed. |

  - Specific end use(s)

| Specific use(s) | : | No information available. |
| --- | --- | --- |

## SECTION 8: Exposure controls/personal protection

  - Control parameters

Contains no substances with occupational exposure limit values.

Derived No Effect Level (DNEL) according to Regulation (EC) No. 1907/2006:

| Substance name | End Use | Exposure routes | Potential health effects | Value |
| --- | --- | --- | --- | --- |
| Benzyl alcohol | Consumers | Oral | Short-term exposure, Acute systemic effects | 20 mg/kg |
|  | Consumers | Oral | Long-term systemic effects | 4 mg/kg |
|  | Consumers | Inhalation | Short-term exposure, Systemic effects | 27 mg/m3 |
|  | Consumers | Inhalation | Long-term systemic effects | 5,4 mg/m3 |
|  | Workers | Inhalation | Short-term exposure, Systemic effects | 110 mg/m3 |

|  | Workers | Inhalation | Long-term systemic effects | 22 mg/m3 |
| --- | --- | --- | --- | --- |
|  | Workers | Dermal | Short-term exposure, Systemic effects | 40 mg/kg |
|  | Workers | Dermal | Long-term systemic effects | 8 mg/kg |

Predicted No Effect Concentration (PNEC) according to Regulation (EC) No. 1907/2006:

| Substance name | Environmental Compartment | Value |
| --- | --- | --- |
| Benzyl alcohol | Soil | 0,456 mg/kg |
|  | Sewage treatment plant | 39 mg/l |
|  | Marine sediment | 0,527 mg/kg |
|  | Marine water | 0,1 mg/l |
|  | Fresh water sediment | 5,27 mg/kg |
|  | Fresh water | 1 mg/l |

  - Exposure controls

Personal protective equipment

| Eye protection | : | Safety glasses with side-shields conforming to EN166 |
| --- | --- | --- |

Hand protection

| Material | : | Nitrile rubber |
| --- | --- | --- |

| Remarks | : | Wear protective gloves. Break through time : > 480 min The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. |
| --- | --- | --- |

| Skin and body protection | : | Choose body protection according to the amount and concentration of the dangerous substance at the work place. |
| --- | --- | --- |
| Respiratory protection | : | In the case of vapour formation use a respirator with an approved filter. Respirator with ABEK filter. |
|  |  | Respirator with a vapour filter (EN 141) |

## SECTION 9: Physical and chemical properties

  - Information on basic physical and chemical properties

| Appearance | : | liquid |
| --- | --- | --- |

| Colour | : | yellow |
| --- | --- | --- |
| Odour | : | None known. |

| Odour Threshold | : | no data available |
| --- | --- | --- |

| pH | : | no data available |
| --- | --- | --- |

| Melting point/range | : | no data available |
| --- | --- | --- |

| Boiling point/boiling range | : | no data available |
| --- | --- | --- |

| Flash point | : | > 96 °C |
| --- | --- | --- |

| Evaporation rate | : | no data available |
| --- | --- | --- |

| Flammability (solid, gas) | : | no data available |
| --- | --- | --- |

| Upper explosion limit | : | no data available |
| --- | --- | --- |

| Lower explosion limit | : | no data available |
| --- | --- | --- |

| Vapour pressure | : | not determined |
| --- | --- | --- |

| Relative vapour density | : | not determined |
| --- | --- | --- |
| Relative density | : | no data available |

Solubility(ies)

| Water solubility | : | slightly soluble |
| --- | --- | --- |

| Partition coefficient: n- octanol/water | : | no data available |
| --- | --- | --- |

| Auto-ignition temperature | : | not determined |
| --- | --- | --- |

| Decomposition temperature | : | no data available |
| --- | --- | --- |

Viscosity

| Viscosity, kinematic | : | not determined |
| --- | --- | --- |

| Explosive properties | : | No hazards to be specially mentioned. |
| --- | --- | --- |

| Oxidizing properties | : | no data available |
| --- | --- | --- |

  - Other information

no data available

## SECTION 10: Stability and reactivity

  - Reactivity

No decomposition if stored and applied as directed.

  - Chemical stability

Stable under recommended storage conditions.

  - Possibility of hazardous reactions

| Hazardous reactions | : | Stable under recommended storage conditions. No hazards to be specially mentioned. |
| --- | --- | --- |

  - Conditions to avoid

| Conditions to avoid | : | Heat |
| --- | --- | --- |

  - Incompatible materials

| Materials to avoid | : | Strong acids and strong bases Oxidizing agents |
| --- | --- | --- |

  - Hazardous decomposition products

Nitrogen oxides (NOx) Carbon oxides

No decomposition if used as directed.

## SECTION 11: Toxicological information

| Acute oral toxicity | : | Acute toxicity estimate: 1 596 mg/kg Method: Calculation method |
| --- | --- | --- |

Information on toxicological effects Acute toxicity

| Acute inhalation toxicity | : | Acute toxicity estimate: 1,67 mg/l Exposure time: 4 h Test atmosphere: dust/mist Method: Calculation method |
| --- | --- | --- |

Skin corrosion/irritation

Remarks: no data available

Serious eye damage/eye irritation

Remarks: no data available

Respiratory or skin sensitisation

Remarks: no data available

Germ cell mutagenicity

| Genotoxicity in vitro | : | Remarks: no data available |
| --- | --- | --- |

| Germ cell mutagenicity- Assessment | : | Not believed to be mutagenic |
| --- | --- | --- |

Carcinogenicity

Result: no data available

| Carcinogenicity - Assessment | : | Not believed to be carcinogenic |
| --- | --- | --- |

Reproductive toxicity

| Effects on fertility | : | Remarks: no data available |
| --- | --- | --- |

| Reproductive toxicity - Assessment | : | Not believed to be reprotoxic |
| --- | --- | --- |

STOT - single exposure

Remarks: no data available

STOT - repeated exposure

Remarks: no data available

Aspiration toxicity

No aspiration toxicity classification

Further information

Remarks: no data available

The following toxicological data refer to:

3-Acetyl-6-methyl-2H-pyran-2,4(3H)-dione (CAS-No.: 520-45-6)

Acute toxicity

| Acute oral toxicity | : | LD50 (Rat): 1 480 mg/kg Method: DOT |
| --- | --- | --- |

Skin corrosion/irritation

Species: Rabbit Exposure time: 4 h Method: DOT

Result: No skin irritation

Serious eye damage/eye irritation

Species: Chicken eye Exposure time: 4 h Assessment: No eye irritation

Method: OECD Test Guideline 438 Result: No eye irritation

GLP: yes

Respiratory or skin sensitisation

Test Type: Local Lymph Node Assay Species: Mouse

Method: OECD Test Guideline 429 Result: not sensitizing

GLP: yes

Germ cell mutagenicity

| Genotoxicity in vitro | : | Test Type: Ames test Species: Salmonella typhimurium Result: negative |
| --- | --- | --- |

Reproductive toxicity

| Effects on fertility | : | Remarks: no data available |
| --- | --- | --- |

STOT - single exposure

Remarks: no data available

STOT - repeated exposure

Remarks: no data available

Further information

Remarks: no data available

Benzyl alcohol (CAS-No.: 100-51-6)

Acute toxicity

| Acute oral toxicity | : | LD50 (Rat): 1 610 mg/kg Remarks: Literary reference |
| --- | --- | --- |

| Acute inhalation toxicity | : | LC50 (Rat): > 4,178 mg/l Exposure time: 4 h Test atmosphere: dust/mist Assessment: The substance or mixture has no acute inhalation toxicity |
| --- | --- | --- |

| Acute dermal toxicity | : | LD50 (Rabbit): 2 000 mg/kg Assessment: The substance or mixture has no acute dermal toxicity Remarks: Literary reference |
| --- | --- | --- |

Skin corrosion/irritation

Species: Rabbit Exposure time: 4 h

Method: OECD Test Guideline 404 Result: No skin irritation

Serious eye damage/eye irritation

Species: Rabbit Assessment: No eye irritation

Method: OECD Test Guideline 405 Result: moderate irritant

Respiratory or skin sensitisation

Test Type: Magnusson & Kligman Species: Guinea pig

Result: not sensitizing Remarks: Literary reference

Germ cell mutagenicity

| Genotoxicity in vitro | : | Test Type: Ames test Result: negative |
| --- | --- | --- |
|  | : | Test Type: Gene mutation Species: mouse lymphoma cells Result: equivocal |
|  | : | Test Type: Chromosome aberration test in vitro Result: positive |

| Genotoxicity in vivo | : | Test Type: In vivo micronucleus test Species: Mouse Application Route: ip |
| --- | --- | --- |

Dose: 50 -100-200 mg/kg Result: negative

Reproductive toxicity

Species: Mouse, female Application Route: Oral Dose: 10d

Fertility: NOAEL: 550 mg/kg food

Repeated dose toxicity

Species: Rat Application Route: Oral

Remarks: Literary reference

Further information

Remarks: May cause sensitisation of susceptible persons by skin contact.

Remarks: Dermal absorption possible

Remarks: High concentration of vapours may induce unconsciousness.

## SECTION 12: Ecological information

  - Toxicity

| Toxicity to fish | : | Remarks: no data available |
| --- | --- | --- |

  - Persistence and degradability

| Biodegradability | : | Result: no data available |
| --- | --- | --- |

  - Bioaccumulative potential

| Bioaccumulation | : | Remarks: no data available |
| --- | --- | --- |

  - Mobility in soil

| Distribution among environmental compartments | : | Remarks: no data available |
| --- | --- | --- |

  - Results of PBT and vPvB assessment

| Assessment | : | This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher. |
| --- | --- | --- |

  - Other adverse effects

| Additional ecological information | : | no data available |
| --- | --- | --- |

The following ecotoxicological data refer to:

3-Acetyl-6-methyl-2H-pyran-2,4(3H)-dione (CAS-No.: 520-45-6)

| Toxicity to fish | : | NOEC (Cyprinus carpio (Carp)): 218 - 415 mg/l Exposure time: 72 h Analytical monitoring: no |
| --- | --- | --- |

| Biodegradability | : | Test Type: OECD Coupled Units Concentration: 12 mg/l Result: biologically well degradable Biodegradation: 99 % Method: OECD Test Guideline 303 A GLP: no |
| --- | --- | --- |
|  |  | Test Type: Zahn-Wellens Test Concentration: 400 mg/l Result: biologically well degradable Biodegradation: 96 % Exposure time: 14 d Method: OECD Test Guideline 302 B |
|  |  | Test Type: Closed Bottle test Concentration: 2 mg/l Result: Readily biodegradable. Biodegradation: 81 % Related to: Theoretical oxygen demand Exposure time: 30 d Method: OECD Test Guideline 301D GLP: no |

| Bioaccumulation | : | Remarks: no data available |
| --- | --- | --- |

| Distribution among environmental compartments | : | Remarks: no data available |
| --- | --- | --- |

| Assessment | : | This substance is not considered to be persistent, bioaccumulating and toxic (PBT)., This substance is not considered to be very persistent and very bioaccumulating (vPvB). |
| --- | --- | --- |

| Additional ecological information | : | No data is available on the product itself. |
| --- | --- | --- |

Benzyl alcohol (CAS-No.: 100-51-6)

| Toxicity to fish | : | LC50 (Leuciscus idus (Golden orfe)): 646 mg/l Exposure time: 48 h Method: DIN 38412 Part 15 Remarks: Literary reference |
| --- | --- | --- |
|  |  | LC50 (Pimephales promelas (fathead minnow)): 460 mg/l Exposure time: 96 h |

| Toxicity to daphnia and other aquatic invertebrates | : | EC50 (Daphnia magna (Water flea)): 400 mg/l Exposure time: 24 h Test Type: Immobilization Method: DIN 38412 L11 Remarks: Literary reference |
| --- | --- | --- |
| Toxicity to algae | : | IC50 (Pseudokirchneriella subcapitata (algae)): 770 mg/l Exposure time: 72 h Method: OECD Test Guideline 201 |
|  |  | NOEC (Pseudokirchneriella subcapitata (algae)): 310 mg/l Exposure time: 72 h Method: OECD Test Guideline 201 |

| Toxicity to daphnia and other aquatic invertebrates (Chronic toxicity) | : | NOEC: 51 mg/l Exposure time: 21 d Species: Daphnia magna (Water flea) Method: OECD Test Guideline 211 |
| --- | --- | --- |

| Toxicity to microorganisms | : | EC50 (Pseudomonas putida): 658 mg/l Exposure time: 16 h Remarks: Literary reference |
| --- | --- | --- |
|  | : | EC50 (Photobacterium phosphoreum): 71 mg/l Exposure time: 30 min Remarks: Literary reference |

| Biodegradability | : | Test Type: Closed Bottle test Result: Readily biodegradable. Biodegradation: > 90,0 % Exposure time: 30 d Method: OECD Test Guideline 301D Remarks: Literary reference |
| --- | --- | --- |

| Bioaccumulation | : | Bioconcentration factor (BCF): 4 Remarks: Literary reference |
| --- | --- | --- |

| Distribution among environmental compartments | : | Adsorption/Soil Remarks: Literary reference |
| --- | --- | --- |

## SECTION 13: Disposal considerations

13.1 Waste treatment methods

| Product | : | Dispose of in accordance with local regulations. Contact waste disposal services. |
| --- | --- | --- |
|  |  | Dispose of contents/container in accordance with local regulation. Contact waste disposal services. Do not dispose of waste into sewer. |
| Contaminated packaging | : | Dispose of as unused product. Do not re-use empty containers. |

## SECTION 14: Transport information

| IATA | Not dangerous goods |
| --- | --- |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | no |

| IMDG | Not dangerous goods |
| --- | --- |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | Marine pollutant: no |
|  |  | : |  |
| ADR |  |  | Not dangerous goods |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | no |
|  |  | : |  |
| RID |  |  | Not dangerous goods |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | no |

| DOT | : | Not dangerous goods |
| --- | --- | --- |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | no |

| TDG | : | Not dangerous goods |
| --- | --- | --- |

| 14.1 | UN number | : | Not applicable |
| --- | --- | --- | --- |
| 14.2 | Proper shipping name | : | Not applicable |
| 14.3 | Transport hazard class | : | Not applicable |
| 14.4 | Packing group | : | Not applicable |
| 14.5 | Environmental hazards | : | no |

| 14.6 | Special precautions for user | : | none |
| --- | --- | --- | --- |

| 14.7 | Transport in bulk according to Annex II of Marpol and the IBC Code | : | Not applicable |
| --- | --- | --- | --- |

SECTION 15: Regulatory information

  - Safety, health and environmental regulations/legislation specific for the substance or mixture

Seveso III: Directive 2012/18/EU of the European Parliament and of the Council on the control of major- accident hazards involving dangerous substances.

Not applicable

| Water contaminating class (Germany) | : | WGK 1 slightly hazardous to water Classification according to AwSV, Annex 1 (5.2) |
| --- | --- | --- |

  - Chemical safety assessment

not required

## SECTION 16: Other information

| Classification of the mixture: |  | Classification procedure: |
| --- | --- | --- |
| Acute Tox. 4 | H302 | Calculation method |
| Acute Tox. 4 | H332 | Calculation method |

Full text of H-Statements

| H302 | : | Harmful if swallowed. |
| --- | --- | --- |
| H332 | : | Harmful if inhaled. |

Full text of other abbreviations

ADN - European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways; ADR - European Agreement concerning the International Carriage of Dangerous Goods by Road; AICS - Australian Inventory of Chemical Substances; ASTM - American Society for the Testing of Materials; bw - Body weight; CLP - Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008; CMR - Carcinogen, Mutagen or Reproductive Toxicant; DIN - Standard of the German Institute for Standardisation; DSL - Domestic Substances List (Canada); ECHA - European Chemicals Agency; EC-Number - European Community number; ECx - Concentration associated with x% response; ELx - Loading rate associated with x% response; EmS - Emergency Schedule; ENCS - Existing and New Chemical Substances (Japan); ErCx - Concentration associated with x% growth rate response; GHS - Globally Harmonized System; GLP - Good Laboratory Practice; IARC - International

Agency for Research on Cancer; IATA - International Air Transport Association; IBC - International Code for the Construction and Equipment of Ships carrying Dangerous Chemicals in Bulk; IC50 - Half maximal inhibitory concentration; ICAO - International Civil Aviation Organization; IECSC - Inventory of Existing Chemical Substances in China; IMDG - International Maritime Dangerous Goods; IMO - International Maritime Organization; ISHL - Industrial Safety and Health Law (Japan); ISO - International Organisation for Standardization; KECI - Korea Existing Chemicals Inventory; LC50 - Lethal Concentration to 50 % of a test population; LD50 - Lethal Dose to 50% of a test population (Median Lethal Dose); MARPOL - International Convention for the Prevention of Pollution from Ships; n.o.s. - Not Otherwise Specified; NO(A)EC - No Observed (Adverse) Effect Concentration; NO(A)EL - No Observed (Adverse) Effect Level; NOELR - No Observable Effect Loading Rate; NZIoC - New Zealand Inventory of Chemicals; OECD - Organization for Economic Co-operation and Development; OPPTS - Office of Chemical Safety and Pollution Prevention; PBT - Persistent, Bioaccumulative and Toxic substance; PICCS - Philippines Inventory of Chemicals and Chemical Substances; (Q)SAR - (Quantitative) Structure Activity Relationship; REACH - Regulation (EC) No 1907/2006 of the European Parliament and of the Council concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals; RID - Regulations concerning the International Carriage of Dangerous Goods by Rail; SADT - Self-Accelerating Decomposition Temperature; SDS - Safety Data Sheet; SVHC - Substance of Very High Concern; TCSI - Taiwan Chemical Substance Inventory; TRGS - Technical Rule for Hazardous Substances; TSCA - Toxic Substances Control Act (United States); UN - United Nations; vPvB - Very Persistent and Very Bioaccumulative

Further information

| Date format | : | dd.mm.yyyy |
| --- | --- | --- |

GB / EN

Changes since the last version are highlighted in the margin. This version replaces all previous versions.

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.